Experiment No. 4

Analysis of Continuous Charge Distribution in Electric Field (Line, Surface, and Volume Charge Density)

Student Name : _AARYA V S_

Register Number : 192512067

# Aim

To write a program for continuous charge distribution in an electric field, covering (a) Linear Charge Density, (b) Surface Charge Density, and (c) Volume Charge Density, using MATLAB.

# Software Required

• MATLAB (R2018 or later)

# Test Case (i)

Line, surface and volume charge density for a given charge, length and radius

MATLAB Program

clc; clear; q=input("Enter the charge value is:"); l=input("Enter the length value is:"); rho_l = q/l; disp('the line charge density is'); disp(rho_l); r=input("Enter the radius value is:"); rho_s = q/(4*pi*r^2); disp('the surface charge density is'); disp(rho_s); rho_v = q/((4/3)*pi*r^3); disp('the volume charge density is'); disp(rho_v); x = linspace(0.1,10,50); y = rho_l./x; z = rho_s./(x.^2); v = rho_v./(x.^3); figure; subplot(1,3,1); plot(x,y,'LineWidth',2); title('Line charge density'); xlabel('Length'); subplot(1,3,2); plot(x,z,'LineWidth',2); title('Surface charge density'); xlabel('Radius'); subplot(1,3,3); plot(x,v,'LineWidth',2); title('Volume charge density'); xlabel('Radius');

Sample Input

Enter the charge value is: 2e-2

Enter the length value is: 1e-3

Enter the radius value is: 2

Actual Program Output (executed in Octave, MATLAB-compatible)

the line charge density is 20 the surface charge density is 3.9789e-04 the volume charge density is 5.9683e-04

Figure 1. Line, surface, and volume charge density fall-off curves

Observation

• All three computed densities (ρₗ=20, ρₛ=3.979×10⁻⁴, ρᵥ=5.968×10⁻⁴) match the manual's theoretical values exactly.

• Each density decreases with the respective geometric factor (1/x, 1/x², 1/x³), consistent with line, surface, and volume scaling.

# Test Case (ii)

Long wire (L = 10 m) carrying uniform charge Q = 50 μC — line charge density and field

MATLAB Program

clc; L = 10; Q = 50e-6; lambda = Q/L; disp("Line charge density:"); disp(lambda); r = linspace(0.1,5,100); k = 9e9; E = k*lambda./r; plot(r,E,'LineWidth',2); xlabel('Distance (m)'); ylabel('Electric Field (N/C)'); title('Electric Field due to Line Charge');

Actual Program Output (executed in Octave, MATLAB-compatible)

Line charge density: 5.0000e-06

Figure 2. Electric field vs distance from a uniformly charged line

Observation

• λ = 5×10⁻⁶ C/m matches the manual's value exactly.

• Field decreases as 1/r, as expected for an infinite line charge.

# Test Case (iii)

Uniformly charged disk (R = 0.5 m, Q = 10 μC) — surface charge density and field

MATLAB Program

clc; R = 0.5; Q = 10e-6; A = pi*R^2; sigma = Q/A; disp("Surface charge density:"); disp(sigma); z = linspace(0.1,2,100); k = 9e9; E = (k*sigma*R^2)./(2*(z.^2 + R^2).^(3/2)); plot(z,E,'LineWidth',2); xlabel('Distance (m)'); ylabel('Electric Field (N/C)'); title('Electric Field due to Charged Disk');

Actual Program Output (executed in Octave, MATLAB-compatible)

Surface charge density: 1.2732e-05

Figure 3. On-axis electric field of a uniformly charged disk

Observation

• σ = 1.2732×10⁻⁵ C/m² matches the manual's value exactly.

• The field falls off non-linearly with axial distance, as expected for a finite charged disk.

# Test Case (iv)

Uniformly charged sphere (R = 2 m, Q = 20 μC) — volume charge density and field inside/outside

MATLAB Program

clc; R = 2; Q = 20e-6; V = (4/3)*pi*R^3; rho = Q/V; disp("Volume charge density:"); disp(rho); r = linspace(0.1,3,100); k = 9e9; E_inside = (k*rho.*r)/3; E_outside = k*Q./(r.^2); plot(r,E_inside,'LineWidth',2); hold on; plot(r,E_outside,'LineWidth',2); xlabel('Distance (m)'); ylabel('Electric Field (N/C)'); title('Electric Field inside and outside sphere'); legend('Inside','Outside');

Actual Program Output (executed in Octave, MATLAB-compatible)

Volume charge density: 5.9683e-07

Figure 4. Electric field inside and outside a uniformly charged sphere

Observation

• ρ = 5.9683×10⁻⁷ C/m³ matches the manual's value exactly.

• The inside curve rises linearly and the outside curve falls as 1/r², both plotted for comparison as intended.

# Results and Observation Table

| Parameters | Observed Result | Theoretical Result | Error |
| --- | --- | --- | --- |
| q=2e-2, l=1e-3 | ρₗ = 20 C/m | ρₗ = 20 C/m | ≈0 |
| q=2e-2, r=2 | ρₛ = 3.979×10⁻⁴ C/m² | ρₛ = 3.98×10⁻⁴ C/m² | ≈0 |
| q=2e-2, r=2 | ρᵥ = 5.968×10⁻⁴ C/m³ | ρᵥ = 5.97×10⁻⁴ C/m³ | ≈0 |
| Q=50μC, L=10m | λ = 5×10⁻⁶ C/m | λ = 5×10⁻⁶ C/m | ≈0 |
| Q=10μC, R=0.5 | σ = 1.273×10⁻⁵ C/m² | σ = 1.27×10⁻⁵ C/m² | ≈0 |
| Q=20μC, R=2 | ρ = 5.968×10⁻⁷ C/m³ | ρ = 5.97×10⁻⁷ C/m³ | ≈0 |

All "Observed Result" values were generated by actually executing the manual's MATLAB code in GNU Octave; all six rows matched the manual's printed values.

# Result

Thus, the program to determine continuous charge distribution in an electric field — covering linear, surface, and volume charge density — was written and executed successfully using MATLAB.