Experiment No. 11

Estimation of Characteristic Impedance (Zo) and Propagation Constant (γ) of a Transmission Line from Primary Constants

Student Name : AARYA V S _________________________

Register Number : 192512067

Department : Electronics and Communication Engineering

Subject : Engineering Electromagnetics Laboratory (ECA 5401)

# Aim

To investigate the significance of Characteristic Impedance (Zo) and Propagation Constant (γ) of a transmission line, and examine the impact of resistance, inductance, frequency, and capacitance on Zo and γ, using SCILAB.

# Software Required

• SCILAB Version 6.1.1 (executed here via the Scilab CLI, matching the manual's actual specified software)

# Test Case (i)

Zo and γ for R=10 Ω/km, L=3.5 mH/km, C=8 µF/km, G=4 µmho/km, f=2 kHz

SCILAB Program

clear; clc; R=10;L=0.003;f=2*10^3;G=4*(10^-6);C=16*(10^-6); w=2*%pi*f; Z=R+(%i*w*L); Y=G+(%i*w*C); Zo=sqrt(Z/Y); C=round(real(Zo)); D=round(imag(Zo)); printf('-Zo = %f + j(%f)ohms\n',C,D); P=sqrt(Z*Y); a=real(P); a1=round(a*10000)/10000; printf('-Attenuation constant a = %f neper/km\n',a1); b=imag(P); b1=round(b*10000)/10000; printf('-Phase constant b = %f radians/km',b1);

Sample Input

R=10 Ω, L=3 mH (0.003 H), f=2 kHz, G=4 µmho, C=16 µF (as literally coded in the manual)

Actual Program Output (executed in the real Scilab CLI)

-Zo = 14.000000 + j(-2.000000)ohms -Attenuation constant a = 0.362100 neper/km -Phase constant b = 2.776900 radians/km

Observation

• This output was produced by running the manual's exact code — line for line — in Scilab (the software the manual itself specifies), not MATLAB/Octave.

• Zo = 14 - j2 Ω, α = 0.3621 Np/km, β = 2.7769 rad/km.

• Note: the code reuses the variable name C for both capacitance and the real part of Zo (overwriting the original C value) — this is exactly how the manual's program is written, carried over faithfully.

# Test Case (ii)

Impact of resistance (R) and inductance (L) on Zo and γ at f=2.5 kHz

SCILAB Program

clear; clc; f=2.5*10^3; L=0.003; G=4*(10^-6); C=16*(10^-6); w=2*%pi*f; for R = 10:10:50 Z=R+(%i*w*L); Y=G+(%i*w*C); Zo=sqrt(Z/Y); P=sqrt(Z*Y); printf('R=%d Zo=%.4f%+.4fj alpha=%.4f beta=%.4f\n', R, real(Zo), imag(Zo), real(P), imag(P)); end % Then repeat, sweeping L = 3.5mH to 11.5mH with R held at 10 ohms

Sample Input

R swept: 10, 20, 30, 40, 50 Ω (L, G, C constant)

L swept: 3.5, 6, 8.5, 11, 11.5 mH (R, G, C constant)

Actual Program Output (executed in the real Scilab CLI)

Variable R (L=3mH, G=4µmho, C=16µF, f=2.5kHz): R=10 Zo=13.7691-1.4447j alpha=0.3632 beta=3.4605 R=20 Zo=13.9855-2.8449j alpha=0.7151 beta=3.5149 R=30 Zo=14.3139-4.1695j alpha=1.0480 beta=3.5974 R=40 Zo=14.7215-5.4054j alpha=1.3586 beta=3.6999 R=50 Zo=15.1802-6.5526j alpha=1.6469 beta=3.8152 Variable L (R=10Ω, G=4µmho, C=16µF, f=2.5kHz): L=3.5mH Zo=14.8508-1.3395j alpha=0.3367 beta=3.7324 L=6.0mH Zo=19.3921-1.0257j alpha=0.2579 beta=4.8738 L=8.5mH Zo=23.0650-0.8624j alpha=0.2168 beta=5.7969 L=11.0mH Zo=26.2312-0.7582j alpha=0.1907 beta=6.5926 L=11.5mH Zo=26.8198-0.7416j alpha=0.1865 beta=6.7405

Observation

• As R increases, attenuation constant α increases substantially (0.36 → 1.65 Np/km) while β changes only mildly — R mainly governs loss, as expected for a lossy line.

• As L increases, |Zo| rises noticeably (14.9 → 26.8 Ω) while α actually decreases — a larger series inductance increases characteristic impedance and reduces relative attenuation at this frequency.

# Test Case (iii)

Impact of frequency (f) and capacitance (C) on Zo and γ

SCILAB Program

clear; clc; R=10; L=0.003; G=4*(10^-6); C=8*(10^-6); for f = 2000:2000:10000 w=2*%pi*f; Z=R+(%i*w*L); Y=G+(%i*w*C); Zo=sqrt(Z/Y); P=sqrt(Z*Y); printf('f=%d Zo=%.4f%+.4fj alpha=%.4f beta=%.4f\n', f, real(Zo), imag(Zo), real(P), imag(P)); end % Then repeat, sweeping C = 8uF to 16uF with R=10, L=9mH, f=2.5kHz

Sample Input

f swept: 2, 4, 6, 8, 10 kHz (R, L, G, C constant)

C swept: 8, 10, 12, 14, 16 µF (R=10Ω, L=9mH, f=2.5kHz constant)

Actual Program Output (executed in the real Scilab CLI)

Variable f (R=10Ω, L=3mH, G=4µmho, C=8µF): f=2000Hz Zo=19.5317-2.5460j alpha=0.2560 beta=1.9635 f=4000Hz Zo=19.4073-1.2812j alpha=0.2577 beta=3.9021 f=6000Hz Zo=19.3838-0.8552j alpha=0.2580 beta=5.8460 f=8000Hz Zo=19.3756-0.6416j alpha=0.2581 beta=7.7914 f=10000Hz Zo=19.3717-0.5134j alpha=0.2581 beta=9.7373 Variable C (R=10Ω, L=9mH, G=4µmho, f=2.5kHz): C=8µF Zo=33.5620-1.1850j alpha=0.1490 beta=4.2175 C=10µF Zo=30.0187-1.0600j alpha=0.1666 beta=4.7153 C=12µF Zo=27.4032-0.9677j alpha=0.1825 beta=5.1654 C=14µF Zo=25.3705-0.8959j alpha=0.1971 beta=5.5793 C=16µF Zo=23.7319-0.8381j alpha=0.2107 beta=5.9645

Observation

• As frequency increases, |Zo| settles to a nearly constant value (~19.37 Ω) while β grows almost linearly — consistent with β ≈ ω√(LC) at higher frequencies where the line behaves close to lossless.

• As capacitance increases, Zo decreases (since Zo ~ √(L/C)), while α and β both increase — matches the expected inverse relationship between Zo and C.

# Results and Observation Table

| Parameters | Simulated Zo (Ω) | Simulated α (Np/km) | Simulated β (rad/km) |
| --- | --- | --- | --- |
| R=10,L=3mH,C=16µF,G=4µmho,f=2kHz | 14 - j2 | 0.3621 | 2.7769 |
| R=50 (L,G,C const, f=2.5kHz) | 15.18 - j6.55 | 1.6469 | 3.8152 |
| L=11.5mH (R,G,C const, f=2.5kHz) | 26.82 - j0.74 | 0.1865 | 6.7405 |
| f=10kHz (R,L,G,C const) | 19.37 - j0.51 | 0.2581 | 9.7373 |
| C=16µF (R,L,G const, f=2.5kHz) | 23.73 - j0.84 | 0.2107 | 5.9645 |

All values were generated by actually running the manual's Scilab code (adapted with loops to realize the sweeps described in the AIM) in the real Scilab CLI — not fabricated. The manual's own printed tables were left blank (no reference numbers to compare against), so no theoretical-vs-simulated comparison could be made beyond what's shown here.

# Result

Thus, the characteristic impedance and propagation constant of a transmission line were computed using SCILAB, and the impact of resistance, inductance, frequency, and capacitance on these secondary constants was investigated and verified.