Experiment No. 12

Analysis of Standing Wave Ratio (SWR) of Transmission Lines by Varying (a) Load Impedance and (b) Characteristic Impedance

Student Name : AARYA V S

Register Number : 19251067

Department : Electronics and Communication Engineering

Subject : Engineering Electromagnetics Laboratory (ECA 5401)

# Aim

To analyze the SWR of a transmission line for a fixed characteristic impedance as the load impedance is varied, and for a fixed load impedance as the characteristic impedance is varied, verifying that SWR = 1 when ZL = Zo, using SCILAB.

# Software Required

• SCILAB Version 6.1.1 (executed here via the Scilab CLI)

# Test Case (i)

SWR for fixed Zo = 50 Ω, load impedance ZL varied across 25, 50, 75, 100 Ω

SCILAB Program

clc; clear; Zo=50; ZLvals = [25 50 75 100]; for i=1:length(ZLvals) ZR = ZLvals(i); K=(ZR-Zo)/(ZR+Zo); ampK=abs(K); S=(1+ampK)/(1-ampK); printf("ZL=%d ohms -> SWR = %f\n", ZR, round(S*1000)/1000); end

Sample Input

Zo = 50 Ω (fixed)

ZL = 25, 50, 75, 100 Ω

Actual Program Output (executed in the real Scilab CLI)

ZL=25 ohms -> SWR = 2.000000 ZL=50 ohms -> SWR = 1.000000 ZL=75 ohms -> SWR = 1.500000 ZL=100 ohms -> SWR = 2.000000

Observation

• When ZL = Zo = 50 Ω, SWR = 1 exactly, confirming the no-reflection condition as the manual expects.

• SWR rises symmetrically as ZL departs from Zo in either direction (25 Ω and 100 Ω both give SWR = 2), since the mismatch magnitude |ZL - Zo|/(ZL + Zo) is what drives the reflection coefficient.

# Test Case (ii)

SWR for fixed ZL = 75 Ω, characteristic impedance Zo varied across 25, 50, 75, 100 Ω

SCILAB Program

clc; clear; ZL=75; Zovals = [25 50 75 100]; for i=1:length(Zovals) Zo = Zovals(i); K=(ZL-Zo)/(ZL+Zo); ampK=abs(K); S=(1+ampK)/(1-ampK); printf("Zo=%d ohms -> SWR = %f\n", Zo, round(S*1000)/1000); end

Sample Input

ZL = 75 Ω (fixed)

Zo = 25, 50, 75, 100 Ω

Actual Program Output (executed in the real Scilab CLI)

Zo=25 ohms -> SWR = 3.000000 Zo=50 ohms -> SWR = 1.500000 Zo=75 ohms -> SWR = 1.000000 Zo=100 ohms -> SWR = 1.333000

Observation

• When Zo = ZL = 75 Ω, SWR = 1 exactly, again confirming the matched condition.

• SWR is not symmetric here (3.0 at Zo=25 vs 1.333 at Zo=100) because the mismatch ratio (ZL-Zo)/(ZL+Zo) is not symmetric about the ZL=75 point in the same way — this is expected, not an error.

# Results and Observation Table

| Case | Parameter Swept | Fixed Value | SWR values obtained |
| --- | --- | --- | --- |
| (i) | ZL = 25, 50, 75, 100 Ω | Zo = 50 Ω | 2.0, 1.0, 1.5, 2.0 |
| (ii) | Zo = 25, 50, 75, 100 Ω | ZL = 75 Ω | 3.0, 1.5, 1.0, 1.333 |

Both sets of values were generated by actually running the manual's SWR formula in Scilab. Note: the manual's Test Case (i) header text (which mentions sweeping ZL from 650-850 Ω using a complex load) is inconsistent with its own AIM statement (which specifies real ZL values 25/50/75/100 Ω against Zo=50 Ω); the AIM's simpler, fully-specified values were used here since they match what the program can actually be run against without inventing extra numbers.

# Result

Thus, the SWR of a transmission line was analyzed for a fixed characteristic impedance as the load impedance was varied, and for a fixed load impedance as the characteristic impedance was varied — in both cases confirming SWR = 1 when Zo = ZL, using SCILAB.