Experiment No. 16

Inference of Q Factor of Rectangular Cavity Resonator for TE101 Mode by Varying Dimensions a, b, c

Student Name : _AARYA V S___

Register Number : 192512067

Department : Electronics and Communication Engineering

Subject : Engineering Electromagnetics Laboratory (ECA 5401)

# Aim

To determine the Q factor of a rectangular cavity resonator operating in the TE101 mode by varying its three dimensions (a, b, c), using SCILAB.

# Software Required

• SCILAB Version 6.1.1 (executed here via the Scilab CLI)

# Test Case (i)

Q factor with a=4.0 cm, b=3.0 cm fixed, c varied from 8 to 12 cm

SCILAB Program

clear; clc; Uo=4*%pi*10^-7; C=5.8*10^7; f101=3.335*10^9; d=sqrt(1/(%pi*f101*Uo*C)); function Q = qfactor(a,b,c) Q=(a*a+c*c)*a*b*c/(d*(2*b*(a^3 + c^3)+a*c*(a*a+c*c))); endfunction for c = [8 9 10 11 12]/100 Q = qfactor(0.04, 0.03, c); disp(Q,'Quality factor of TE101 = '); end

Sample Input

a=4.0 cm, b=3.0 cm (fixed); c = 8, 9, 10, 11, 12 cm; copper cavity (σ=5.8×10⁷ S/m)

Actual Program Output (executed in the real Scilab CLI)

c=8.0 cm -> Q=11155.7 c=9.0 cm -> Q=11096.4 c=10.0 cm -> Q=11034.2 c=11.0 cm -> Q=10975.7 c=12.0 cm -> Q=10923.2

Observation

• Q decreases slowly and monotonically as c increases from 8 to 12 cm (a drop of only ~2% across the range) — the resonator's Q is much less sensitive to c than to a or b, as seen in the later test cases.

# Test Case (ii)

Finding the dimension combination that maximizes Q

SCILAB Program

clear; clc; // Same qfactor() function as Test Case (i) best=0; for a=[4 5 6 7]/100 for b=[3 4 5]/100 for c=[8 9 10 11 12]/100 Q=qfactor(a,b,c); if Q>best then best=Q; besta=a; bestb=b; bestc=c; end end end end

Sample Input

Search grid: a=4–7 cm, b=3–5 cm, c=8–12 cm (the ranges explored across Test Cases 1, 3, and 4)

Actual Program Output (executed in the real Scilab CLI)

Maximum Q = 19185.1 at a = 7.0 cm, b = 5.0 cm, c = 12.0 cm

Observation

• The largest dimensions in the search grid give the highest Q — Q increases with overall cavity volume, since larger a and b (which both appear with high power in the numerator relative to the surface-loss term in the denominator) reduce the surface-to-volume ratio and hence reduce relative wall losses.

• This matches the general physical intuition: bigger, lower-loss cavities store energy more efficiently relative to their conductive wall losses, giving higher Q.

# Test Case (iii)

Sensitivity of Q to dimension a (b=4 cm, c=10 cm fixed, a swept 4.0–6.0 cm)

SCILAB Program

clear; clc; // Same qfactor() function for a = [4.0 4.5 5.0 5.5 6.0]/100 Q = qfactor(a, 0.04, 0.10); end

Sample Input

b=4.0 cm, c=10.0 cm (fixed); a = 4.0, 4.5, 5.0, 5.5, 6.0 cm

Actual Program Output (executed in the real Scilab CLI)

a=4.0 -> Q=12331.8 a=4.5 -> Q=13376.5 a=5.0 -> Q=14325.6 a=5.5 -> Q=15180.9 a=6.0 -> Q=15945.2

Observation

• Q rises substantially with a (~29% increase from a=4 to a=6 cm) — much more sensitive than the c-sweep in Test Case (i).

• In practical resonator design, a (and similarly b) is generally the more critical dimension to optimize for high Q, since it more strongly affects the wall-loss-to-volume ratio for the TE101 mode; c has comparatively little leverage.

# Test Case (iv)

Sensitivity of Q to dimension b (a=5 cm, c=10 cm fixed, b swept 3.0–5.0 cm)

SCILAB Program

clear; clc; // Same qfactor() function for b = [3.0 3.5 4.0 4.5 5.0]/100 Q = qfactor(0.05, b, 0.10); end

Sample Input

a=5.0 cm, c=10.0 cm (fixed); b = 3.0, 3.5, 4.0, 4.5, 5.0 cm

Actual Program Output (executed in the real Scilab CLI)

b=3.0 -> Q=12603.7 b=3.5 -> Q=13533.2 b=4.0 -> Q=14325.6 b=4.5 -> Q=15009.0 b=5.0 -> Q=15604.6

Observation

• Q rises with b as well (~24% increase from b=3 to b=5 cm), confirming that both a and b are the dimensions worth prioritizing when optimizing for Q, consistent with Test Case (ii)'s finding that the largest a and b in the grid gave the maximum Q.

# Results and Observation Table

| Dimensions Varied | Range | Q Factor Range | Sensitivity |
| --- | --- | --- | --- |
| c (a=4, b=3 fixed) | 8–12 cm | 11155.7 → 10923.2 | Low (~2% change) |
| a (b=4, c=10 fixed) | 4.0–6.0 cm | 12331.8 → 15945.2 | High (~29% change) |
| b (a=5, c=10 fixed) | 3.0–5.0 cm | 12603.7 → 15604.6 | High (~24% change) |
| Best combination found | a=7,b=5,c=12 cm | Q = 19185.1 | Maximum in search grid |

All Q values were computed by actually running the manual's Scilab Q-factor formula (with sweeps added to realize each test case's AIM) in the real Scilab CLI.

# Result

Thus, the Q factor of a rectangular cavity resonator operating in the TE101 mode was determined for varying dimensions a, b, and c using SCILAB, showing that a and b have a much stronger influence on Q than c, and that larger overall cavity dimensions yield higher Q.