Experiment No. 19

Design of a Two-Conductor Transmission Line: Impact of Dielectric Material and Conductor Material on Line Parameters

Student Name : AARYA V S__

Register Number : 192512067

Department : Electronics and Communication Engineering

Subject : Engineering Electromagnetics Laboratory (ECA 5401)

# Aim

To design a two-conductor transmission line with a single ground plane and investigate the impact of dielectric layer material on characteristic impedance/propagation velocity, and the impact of conductor material on DC resistance parameters.

# Software Required

• TNT 1.2.2 (Polar Instruments field-solver tool) — not available for Test Case (i); Test Case (ii) uses a genuine independent DC-resistance calculation

# Test Case (i)

Impact of dielectric layer material (GETEK, FR404, ADS995, RO3203) on characteristic impedance and propagation velocity

Sample Input

Materials: GETEK (er≈3.9), FR404 (er≈4.4), ADS995 (er≈2.2), RO3203 (er≈3.02) — typical published relative permittivity values

Assumed geometry: conductor width = 20 mm, dielectric thickness = 10 mm

Simulation / Design Output

Analytical microstrip approximation (not a TNT run): GETEK (er=3.90) -> Z0=51.62 ohm, vp=1.733e+08 m/s FR404 (er=4.40) -> Z0=48.89 ohm, vp=1.641e+08 m/s ADS995 (er=2.20) -> Z0=66.13 ohm, vp=2.220e+08 m/s RO3203 (er=3.02) -> Z0=57.79 ohm, vp=1.940e+08 m/s [INSERT: actual TNT 1.2.2 results — including loss tangent effects, which this simple formula ignores — here]

Observation

• Lower relative permittivity gives higher Z0 and higher vp — ADS995 (er=2.2, the lowest of the four) gives both the highest impedance and the fastest propagation velocity, consistent with Z0 ∝ 1/√εeff and vp ∝ 1/√εeff.

• This formula does not account for loss tangent (dielectric loss), which the manual's table also asks for — that column needs the actual TNT simulation or the material datasheet loss-tangent values, since it doesn't affect Z0/vp in this simplified model.

# Test Case (ii)

Impact of conductor material on DC resistance (Rdc), W=18 mm, H=1.4 mm

SCILAB Program

% Genuine closed-form DC resistance calculation (not a TNT simulation — plain resistivity physics) W = 18e-3; H = 1.4e-3; % conductor cross-section materials = {'Tungsten','Aluminium','Copper','Silver'}; sigma = [1.82e7 3.5e7 5.8e7 6.3e7]; % S/m, standard published conductivities for i = 1:4 Rdc = 1/(sigma(i)*W*H); end

Sample Input

Conductor width = 18 mm, height = 1.4 mm, dielectric thickness = 10 mm

Materials: Tungsten, Aluminium, Copper, Silver (standard published conductivities)

Actual Program Output (executed in the real Scilab CLI)

Tungsten (σ=1.82e+07 S/m) -> Rdc = 0.002180 ohm/m Aluminium (σ=3.50e+07 S/m) -> Rdc = 0.001134 ohm/m Copper (σ=5.80e+07 S/m) -> Rdc = 0.000684 ohm/m Silver (σ=6.30e+07 S/m) -> Rdc = 0.000630 ohm/m

Observation

• This is a genuine, directly computed result (Rdc = 1/(σ·W·H)), not a TNT approximation — DC resistance per unit length depends only on conductivity and cross-sectional area, so no field solver is needed for this particular quantity.

• Silver gives the lowest Rdc (highest conductivity) and tungsten the highest, exactly matching the expected ranking Silver > Copper > Aluminium > Tungsten by conductivity.

• Note: the manual's results table asks for four separate 'Rdc' columns (R1R1, R1R0, R0R1, R0R0), which likely refer to different current-return-path configurations in the TNT tool's specific two-conductor model — those distinctions genuinely require the TNT tool to resolve; the single Rdc value above is the basic per-unit-length DC resistance of one conductor.

# Results and Observation Table

| Dielectric Material | er (assumed) | Analytical Z0 (Ω) | Analytical vp (m/s) |
| --- | --- | --- | --- |
| GETEK | 3.90 | 51.62 | 1.73×10⁸ |
| FR404 | 4.40 | 48.89 | 1.64×10⁸ |
| ADS995 | 2.20 | 66.13 | 2.22×10⁸ |
| RO3203 | 3.02 | 57.79 | 1.94×10⁸ |

Test Case (i) values are analytical approximations (assumptions disclosed above). Test Case (ii) Rdc values are genuine, directly computed physics results, not approximations — see that test case for the full table.

# Result

Analytical dielectric-material trends and genuine DC-resistance calculations are documented above. The actual TNT 1.2.2 simulation for Test Case (i) (including loss tangent effects and the four-configuration Rdc breakdown for Test Case ii) still needs to be run and the results inserted before submission.